TREE-ELEMENT YAGI

(20; 15; 10 MHz)

The order of assembly and connection of antenna

(tuned in before).

- Assemble the boom by the way of installation of its parts to corresponding joint hole. Then to fix them. The places of installations of the elements (director, reflector and vibrator) are marked by a marker.

- Assemble the radiate elements by the way of installation of the short elements with the traps to corresponding carrying elements concerning the mark of marker. Then cover U-bolts and fix them. Ventilation holes in the traps must be situated strictly downwards. Stand the subelement with a dielectric racks below the vibrator. Stand and fix it when the element is not weighed down.

- Then fasten the radiate elements on the boom accordingly marks of marker. Fix them with U-bolts.

- Cymmetryment makes by the next way: wind eight whorls of cable around 15 cm whorl to the whorl. An area of section of cable must be a minimum length.

- Sags of the elements must be correcting by a kapron line stretching.

- Stove of fastening of antenna to reductor allows two variants:

- flatness fastening;

- fastening on the rack.

## CHART OF ANTENNA

185 cm

233 cm

Mark – 1 - director

Mark – 2 - vibrator

Mark – 3 - reflector

Fastening of mast

1

## From above view

Assembly of director and reflector

Assembly of vibrator;

symmetrymet

15 cm

Kapron line

Kapron line

## Front view

8 whorls of cable